BÁO CÁO CUỐI KỲ

IT3280 – 147793 – THỰC HÀNH KIẾN TRÚC MÁY TÍNH

| Họ và tên | Lê Đồng Cảnh Phú |
| --- | --- |
| Mã số sinh viên | 20225755 |

# Bài 10: Máy tính bỏ túi

- Mô tả bài toán: Sử dụng 2 ngoại vi là bàn phím và led 7 thanh để xây dựng một máy tính bỏ túi đơn giản. Hỗ trợ các phép toán +, -, *, /. Do trên bàn phím không có các phím trên nên sẽ dùng các phím

- Bấm phím a để nhập phép tính +

- Bấm phím b để nhập phép tính –

- Bấm phím c để nhập phép tính *

- Bấm phím d để nhập phép tính /

- Bấm phím f để nhập phép =

Yêu cầu cụ thể như sau:

- Khi nhấn các phím số, hiển thị lên LED, do chỉ có 2 LED nên chỉ hiện thị 2 số cuối cùng. Ví dụ khi nhấn phím 1 → hiện thị 01. Khi nhấn thêm phím 2 → hiển thị 12. Khi nhấn thêm phím 3 → hiển thị 23.

- Sau khi nhập số, sẽ nhập phép tính + - * /

- Sau khi nhấn phím f (dấu =) , tính toán và hiển thị kết quả lên LED

- Ý tưởng: Đẩy các số hiển thị trên LED tương ứng của phím đã nhập vào trong stack, khi gọi toán tử sẽ lưu số thứ nhất vào thanh ghi $s4, tương tự với số thứ hai lưu vào thanh ghi $s5.

- Bài làm

.eqv	SEVENSEG_LEFT	0xFFFF0011	# Dia chi cua den led 7 doan trai.

# 	bit 0 = doan a;

# 	bit 1 = doan b;

#	bit 7 = dau .

.eqv	SEVENSEG_RIGHT	0xFFFF0010	# Dia chi cua den led 7 doan phai.

.eqv 	IN_ADDRESS_HEXA_KEYBOARD 	0xFFFF0012

.eqv 	OUT_ADDRESS_HEXA_KEYBOARD 	0xFFFF0014

.data

zero:  .byte 0x3f

one:   .byte 0x6

two:   .byte 0x5b

three: .byte 0x4f

four:  .byte 0x66

five:  .byte 0x6d

six:   .byte 0x7d

seven: .byte 0x7

eight: .byte 0x7f

nine:  .byte 0x6f

mess1: .asciiz "Khong the chia cho so 0 \n"

mess2: .asciiz "Khong the hien thi ket qua am \n"

.text

main:

Khoi_tao:

li 	$t0, SEVENSEG_LEFT     	#Bien gia tri so cua den LED trai

li 	$t5, SEVENSEG_RIGHT     #Bien gia tri so cua den LED phai

li 	$s0, 0      		#Bien kiem tra loai bien nhap vao: (0: so), (1 :toan tu),(2: exit)

li 	$s1, 0     		#so hien thi o led phai

li 	$s2, 0   		#so hien thi o led trai

li 	$s3, 0     		#bien kiem tra loai toan tu (1: +, 2: -, 3: *, 4: /)

li 	$s4, 0      		#so thu nhat

li 	$s5, 0   		#so thu hai

li 	$s6, 0     		#ket qua phep tinh: '+' , '-', '*', '/'

li 	$t9, 0 		#gia tri tam thoi

li 	$t1, IN_ADDRESS_HEXA_KEYBOARD  	#bien dieu khien hang keyboard va enable keyboard interrupt

li 	$t2, OUT_ADDRESS_HEXA_KEYBOARD 	#bien chua vi tri key nhap vao hang va cot

li 	$t3, 0x80			# bit dung enable keyboard interrupt va enable kiem tra tung hang keyboard

sb 	$t3, 0($t1)

li 	$t7, 0       		#gia tri cua so hien tren led

li 	$t4, 0			#byte hien thi len led, zero->nine

First_value:

li 	$t7, 0        		#gia tri cua bit can hien thi ban dau

addi 	$sp,$sp, 4		#day vao stack

sb 	$t7, 0($sp)

lb 	$t4, zero 		#bit dau tien can hien thi

addi 	$sp, $sp, 4  	#day vao stack

sb 	$t4, 0($sp)

Loop1:

beq $s0, 2, end_main

nop

nop

nop

nop

b 	Loop1		#Wait for interrupt

nop

nop

nop

nop

b 	Loop1		#Wait for interrupt

nop

nop

nop

nop

b 	Loop1		#Wait for interrupt

end_loop1:

end_main:

li 	$v0, 10

syscall

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~ ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

#--------------------------------------------------------

# Processing: Neu co o nao duoc nhap thi chuyen toi hang tuong ung voi o do

#--------------------------------------------------------

process:

jal check_row1 #Kiem tra hang 1 xem co phim nao duoc nhap hay khong

bnez $t3, convert_row1 #t3!=0 -> co phim duoc nhap, kiem tra cac phim trong hang

nop

jal check_row2

bnez $t3, convert_row2

nop

jal check_row3

bnez $t3, convert_row3

nop

jal check_row4

bnez $t3, convert_row4

nop

#---------------------------------------

#Kiem tra xem phim duoc nhap o hang nao

#---------------------------------------

#--------------------------------------------------------------------

# 			col 0x1 		col 0x2		col 0x4 		col 0x8

#

# row 0x1 		0 			1 			2 			3

# 			0x11 			0x21 			0x41 			0x81

#

# row 0x2 		4 			5 			6 			7

# 			0x12 			0x22 			0x42 			0x82

#

# row 0x4 		8 			9 			a 			b

# 			0x14 			0x24 			0x44 			0x84

#

# row 0x8 		c 			d 			e 			f

#			0x18 			0x28 			0x48 			0x88

#

#---------------------------------------------------------------------

check_row1:

addi $sp, $sp, 4

sw $ra, 0($sp) #luu $ra vi chung ta co the thay doi no sau

li $t3, 0x81 #Kich hoat interrupt, cho phep bam phim o hang 1

sb $t3, 0($t1)

jal get_value #Lay vi tri cua phim duoc nhap (neu co)

lw $ra, 0($sp)

addi $sp, $sp, -4

jr $ra

check_row2:

addi $sp, $sp, 4

sw $ra, 0($sp) #luu $ra vi chung ta co the thay doi no sau

li $t3, 0x82 #Kich hoat interrupt, cho phep bam phim o hang 2

sb $t3, 0($t1)

jal get_value #Lay vi tri cua phim duoc nhap (neu co)

lw $ra, 0($sp)

addi $sp, $sp, -4

jr $ra

check_row3:

addi $sp, $sp, 4

sw $ra, 0($sp) #luu $ra vi chung ta co the thay doi no sau

li $t3, 0x84 #Kich hoat interrupt, cho phep bam phim o hang 3

sb $t3, 0($t1)

jal get_value #Lay vi tri cua phim duoc nhap (neu co)

lw $ra, 0($sp)

addi $sp, $sp, -4

jr $ra

check_row4:

addi $sp, $sp, 4

sw $ra, 0($sp) #luu $ra vi chung ta co the thay doi no sau

li $t3, 0x88 #Kich hoat interrupt, cho phep bam phim o hang

sb $t3, 0($t1)

jal get_value #Lay vi tri cua phim duoc nhap (neu co)

lw $ra, 0($sp)

addi $sp, $sp, -4

jr $ra

#------------------------------------------------------

#Lay gia tri cua phim vua duoc nhap

#------------------------------------------------------

get_value:

addi $sp, $sp, 4

sw $ra, 0($sp)

li $t2, OUT_ADDRESS_HEXA_KEYBOARD #dia chi chua vi tri phim vua nhap

lb $t3, 0($t2) #load vi tri phim vua nhap

lw $ra, 0($sp)

addi $sp, $sp, -4

jr $ra

#-----------------------------------------------------

#Chuyen doi tu vi tri sang bit

#-----------------------------------------------------

convert_row1:

beq $t3, 0x11, case_0 #0x11 -> so 0

beq $t3, 0x21, case_1 #0x21 -> so 1

beq $t3, 0x41, case_2 #0x41 -> so 2

beq $t3, 0xffffff81, case_3 #0xffffff81 -> so 3

case_0:

lb $t4, zero #t4=0x3f = 0 tren digital lab sim

li $t7,0 #t7=0

j update

case_1:

lb $t4, one

li $t7, 1

j update

case_2:

lb $t4, two

li $t7, 2

j update

case_3:

lb $t4, three

li $t7, 3

j update

convert_row2:

beq $t3, 0x12, case_4 #0x12 -> so 4

beq $t3, 0x22, case_5 #0x22 -> so 5

beq $t3, 0x42, case_6 #0x42 -> so 6

beq $t3, 0xffffff82, case_7 #0xffffff82 -> so 7

case_4:

lb $t4, four #t4=0x66 = 4 tren digital lab sim

li $t7,4 #t7=4

j update

case_5:

lb $t4, five

li $t7, 5

j update

case_6:

lb $t4, six

li $t7, 6

j update

case_7:

lb $t4, seven

li $t7, 7

j update

convert_row3:

beq $t3, 0x14, case_8 #0x14 -> so 8

beq $t3, 0x24, case_9 #0x24 -> so 9

beq $t3, 0x44, case_a #0x44 -> phim a

beq $t3, 0xffffff84, case_b #0xffffff8f -> phim b

case_8:

lb $t4, eight #t4=0x7f = 8 tren digital lab sim

li $t7,8 #t7=4

j update

case_9:

lb $t4, nine

li $t7, 9

j update

#Truong hop phim a: Phep cong:

case_a:

addi $a3, $zero, 1

addi $s0, $s0, 1 #s0 = 1 -> toan tu duoc nhap vao

addi $s3, $zero, 1 #s3 = 1 -> phep cong

j first_number

#Truong hop phim b: Phep tru

case_b:

addi 	$a3, $zero, 2

addi 	$s0, $s0, 1		#s0 = 1 -> toan tu duoc nhap vao

addi 	$s3, $zero, 2		#s3 = 2 -> phep tru

j 	first_number

convert_row4:

beq 	$t3, 0x18, case_c		# 0x18 -> phim c

beq 	$t3, 0x28, case_d		# 0x28 -> phim d

beq 	$t3, 0x48, case_e		# 0x48 -> phim e

beq 	$t3, 0xffffff88, case_f		# 0xffffff88 -> phim f

#Truong hop phim c: Phep nhan

case_c:

addi 	$a3, $zero, 3

addi 	$s0, $s0, 1          	#s0 = 1 -> toan tu duoc nhap vao

addi 	$s3, $zero, 3		#s3 = 3 -> phep nhan

j first_number        	#chuyen den ham chuyen 2 byte dang hien tren 2 led thanh so de tinh toan

#Truong hop phim d: phep chia

case_d:

addi 	$a3, $zero, 4

addi 	$s0, $s0, 1		#s0 = 1 -> toan tu duoc nhap vao

addi 	$s3, $zero, 4		#s3 = 4 -> phep chia lay phan nguyen

j 	first_number

#Truong hop phim e: Tat chuong trinh

case_e:

li $s0, 2

j finish

#So dau tien hien thi tren led

first_number:

addi $s4, $t9, 0 #Luu so dau tien vao thanh s4

li $t9, 0

j done

#Truong hop phim f:  Dau bang

case_f:

addi $s5, $t9, 0

#So thu hai duoc hien thi tren led

set_second_number:

beq 	$s3, 1, phep_cong	# s3 = 1 -> cong

beq 	$s3, 2, phep_tru	# s3 = 2 -> tru

beq 	$s3, 3, phep_nhan	# s3 = 3 -> nhan

beq 	$s3, 4, phep_chia	# s3 = 4 -> chia

phep_cong:

add $s6, $s5, $s4

li $s3, 0

li $t9, 0

j print_cong

print_cong:

li $v0, 1

move $a0, $s4

syscall

li $s4, 0 #reset so thu nhat

li $v0,  11

li $a0, '+'

syscall

li $v0, 1

move $a0, $s5

syscall

li $s5, 0 #reset so thu hai

li $v0, 11

li $a0, '='

syscall

li $v0, 1

move $a0, $s6

syscall

#move $s4, $s6 #Bien ket qua cua phep tinh truoc lam so thu nhat cua phep tinh sau

nop

li $v0, 11

li $a0, '\n'

syscall

li $s7, 100

div $s6, $s7

mfhi $s6	    	#Lay 2 gia tri cuoi cua ket qua

j hien_thi_ket_qua	#Hien thi ket qua tren led

nop

phep_tru:

sub $s6, $s4, $s5

li 	$s3, 0

li 	$t9, 0

blt $s6, 0, message1

j 	print_tru

nop

message1:

li $v0, 55

la $a0, mess2

li $a1, 0

syscall

j reset_led

print_tru:

li $v0, 1

move $a0, $s4

syscall

li $s4, 0 #reset so thu nhat

li $v0,  11

li $a0, '-'

syscall

li $v0, 1

move $a0, $s5

syscall

li $s5, 0 #reset so thu hai

li $v0, 11

li $a0, '='

syscall

li $v0, 1

move $a0, $s6

syscall

#move $s4, $s6 #Bien ket qua cua phep tinh truoc lam so thu nhat cua phep tinh sau

nop

li $v0, 11

li $a0, '\n'

syscall

li $s7, 100

div $s6, $s7

mfhi $s6	    	#Lay 2 gia tri cuoi cua ket qua

j hien_thi_ket_qua	#Hien thi ket qua tren led

nop

phep_nhan:

mul $s6, $s4, $s5

li $s3, 0

li $t9, 0

j print_nhan

nop

print_nhan:

li $v0, 1

move $a0, $s4

syscall

li $s4, 0 #reset so thu nhat

li $v0,  11

li $a0, '*'

syscall

li $v0, 1

move $a0, $s5

syscall

li $s5, 0 #reset so thu hai

li $v0, 11

li $a0, '='

syscall

li $v0, 1

move $a0, $s6

syscall

#move $s4, $s6 #Bien ket qua cua phep tinh truoc lam so thu nhat cua phep tinh sau

nop

li $v0, 11

li $a0, '\n'

syscall

li $s7, 100

div $s6, $s7

mfhi $s6	    	#Lay 2 gia tri cuoi cua ket qua

j hien_thi_ket_qua	#Hien thi ket qua tren led

nop

phep_chia:

beq $s5, 0, message2

li $s3, 0

div $s4, $s5

mflo $s6

mfhi $s7

li $t9, 0

j print_chia

message2:

li $v0, 55

la $a0, mess1

li $a1, 0

syscall

j reset_led

print_chia:

li $v0, 1

move $a0, $s4

syscall

li $s4, 0 #reset so thu nhat

li $v0,  11

li $a0, '/'

syscall

li $v0, 1

move $a0, $s5

syscall

li $s5, 0 #reset so thu hai

li $v0, 11

li $a0, '='

syscall

li $v0, 1

move $a0, $s6

syscall

#move $s4, $s6 #Bien ket qua cua phep tinh truoc lam so thu nhat cua phep tinh sau

nop

li $v0, 11

li $a0, '\n'

syscall

li $s7, 100

div $s6, $s7

mfhi $s6	    	#Lay 2 gia tri cuoi cua ket qua

j hien_thi_ket_qua	#Hien thi ket qua tren led

nop

#--------------------------------

#Hien thi ket qua tren den led

#So 'ab'

# Den trai = a = ab div 10

# Den phai = b = ab mod 10

#--------------------------------

hien_thi_ket_qua:

li 	$t8, 10			# Gia tri trung gian = 10

div 	$s6, $t8    		# $s6 = a

mflo 	$t7        		# $t7 = result

jal 	check    		#chuyen den ham chuyen t7 thanh bit hien thi len led

sb 	$t4, 0($t0)  		# hien thi len led trai

add 	$sp, $sp, 4

sb 	$t7, 0($sp)		#day gia tri bit nay vao stack

add 	$sp, $sp, 4

sb 	$t4, 0($sp)    		#day bit nay vao stack

add 	$s2, $t7, $zero   	#s2 = gia tri bit led trai

mfhi 	$t7       	#t7 = remainder

jal 	check    	#convert t7 thanh bit hien thi len led

sb 	$t4, 0($t5)  	#hien thi len led phai

add 	$sp, $sp, 4

sb 	$t7, 0($sp)	# day gia tri bit nay vao stack

add 	$sp, $sp, 4

sb 	$t4, 0($sp)    	# day bit nay vao stack

add 	$s1, $t7, $zero	# s1 = gia tri bit led phai

j 	reset_led     	# ham reset lai led

check:

addi 	$sp, $sp, 4

sw 	$ra, 0($sp)

beq 	$t7, 0, check_0		# t7 = 0 -> Hien thi so 0 tren thanh led

beq 	$t7, 1, check_1	   	# t7 = 1 -> Hien thi so 1 tren thanh led

beq 	$t7, 2, check_2		# t7 = 2 -> Hien thi so 2 tren thanh led

beq 	$t7, 3, check_3		# t7 = 3 -> Hien thi so 3 tren thanh led

beq 	$t7, 4, check_4		# t7 = 4 -> Hien thi so 4 tren thanh led

beq 	$t7, 5, check_5		# t7 = 5 -> Hien thi so 5 tren thanh led

beq 	$t7, 6, check_6		# t7 = 6 -> Hien thi so 6 tren thanh led

beq 	$t7, 7, check_7		# t7 = 7 -> Hien thi so 7 tren thanh led

beq 	$t7, 8, check_8		# t7 = 8 -> Hien thi so 8 tren thanh led

beq 	$t7, 9, check_9		# t7 = 9 -> Hien thi so 9 tren thanh led

#----------------------------------------------

#Chuyen gia tri thanh bit hien tren thanh led

#----------------------------------------------

check_0:

lb 	$t4, zero

j 	finish_check

check_1:

lb 	$t4, one

j 	finish_check

check_2:

lb 	$t4, two

j 	finish_check

check_3:

lb 	$t4, three

j	finish_check

check_4:

lb 	$t4, four

j 	finish_check

check_5:

lb 	$t4, five

j 	finish_check

check_6:

lb 	$t4, six

j 	finish_check

check_7:

lb 	$t4, seven

j 	finish_check

check_8:

lb 	$t4, eight

j 	finish_check

check_9:

lb 	$t4, nine

j 	finish_check

finish_check:

lw 	$ra, 0($sp)

addi 	$sp, $sp, -4

jr 	$ra

update:

mul 	$t9, $t9, 10

add 	$t9, $t9, $t7

#-------------------------------------

# Hoan thanh xong 1 so -> reset_led

#-------------------------------------

done:

beq $s0,1,reset_led   # s0 = 1 -> toan tu -> chuyen den ham reset led

nop

#-------------------------------------

#ham hien thi bit len led ben trai

#-------------------------------------

load_to_left_led:

lb 	$t6, 0($sp)       #load bit hien thi led tu stack

add 	$sp, $sp, -4

lb 	$t8, 0($sp)       #load gia tri cua bit nay

add 	$sp, $sp, -4

add 	$s2, $t8, $zero   #s2 = gia tri bit led trai

sb 	$t6, 0($t0)       # hien thi len led trai

#-------------------------------------

#ham hien thi bit len led ben phai

#-------------------------------------

load_to_right_led:

sb 	$t4, 0($t5)       # hien thi bit len led phai

add 	$sp, $sp,4

sb 	$t7, 0($sp)	  #day gia tri bit nay vao stack

add 	$sp, $sp,4

sb 	$t4, 0($sp)       #day bit nay vao stack

add 	$s1, $t7, $zero   #s1 = gia tri bit led phai

j 	finish

reset_led:

li 	$s0, 0           #s0 = 0 -> doi nhap so tiep theo trong 2 so

li 	$t8, 0

addi 	$sp, $sp, 4

sb 	$t8, 0($sp)

lb 	$t6, zero        # day bit zero vao stack

addi 	$sp, $sp, 4

sb 	$t6, 0($sp)

beq 	$a3, 1, set_add

nop

beq 	$a3, 2, set_sub

nop

beq 	$a3, 3, set_mul

nop

beq 	$a3, 4, set_div

nop

set_add:

addi 	$s3, $zero, 1

j 	finish

nop

set_sub:

addi 	$s3, $zero, 2

j 	finish

nop

set_mul:

addi 	$s3, $zero, 3

j  	finish

nop

set_div:

addi 	$s3, $zero, 4

j 	finish

nop

finish:

j end_exception

nop

end_exception:

# return to start of the loop instead of where the interrupt occur, since the loop doesn't do meaningful thing

la $a3, Loop1

mtc0 $a3, $14		#dat thanh ghi $14 thanh gia tri duoc luu tru trong $a3

eret