BÁO CÁO THỰC HÀNH

IT3280 – 147793 – THỰC HÀNH KIẾN TRÚC MÁY TÍNH

Tuần 11: Interrupts & IO programming

| Họ và tên | Lê Đồng Cảnh Phú |
| --- | --- |
| Mã số sinh viên | 20225755 |

# Assignment 1

# Laboratory Exercise 11, Assignment 1

#------------------------------------------------------

# 		col 0x1 col 0x2 col 0x4 col 0x8

#

# row 0x1 	0 	1 	2 	3

# 		0x11 	0x21 	0x41 	0x81

#

# row 0x2 	4 	5 	6 	7

# 		0x12 	0x22 	0x42 	0x82

#

# row 0x4 	8 	9 	a 	b

# 		0x14 	0x24 	0x44 	0x84

#

# row 0x8 	c 	d 	e 	f

# 		0x18 	0x28 	0x48 	0x88

#

#------------------------------------------------------

# command row number of hexadecimal keyboard (bit 0 to 3)

# Eg. assign 0x1, to get key button 0,1,2,3

# assign 0x2, to get key button 4,5,6,7

# NOTE must reassign value for this address before reading,

# eventhough you only want to scan 1 row

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

# receive row and column of the key pressed, 0 if not key pressed

# Eg. equal 0x11, means that key button 0 pressed.

# Eg. equal 0x28, means that key button D pressed.

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.text

main:

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

li $t3, 0x1  			# start with row 1

polling:

sb $t3, 0($t1)  			# assign row to scan

lb $a0, 0($t2)  			# read scan code of key button

beqz $a0, next_row  		# if no key pressed, check next row

print:

li $v0, 34  			# print integer (hexa)

syscall

li $v0, 11

li $a0, ' '

syscall

sleep:

li $a0, 1000  			# sleep 1000ms

li $v0, 32

syscall

next_row:

sll $t3, $t3, 1  			# shift left to check next row

ble $t3, 0x8, polling  		# loop until all rows checked

j main  				# restart from row 1

# Assignment 2

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.data

Message: .asciiz "Key pressed: "

NewLine: .asciiz "\n"

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# MAIN Procedure

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.text

main:

#---------------------------------------------------------

# Enable interrupts you expect

#---------------------------------------------------------

# Enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

Loop:

li $t3, 0x80 	# bit 7 of = 1 to enable interrupt

sb $t3, 0($t1)

#---------------------------------------------------------

# No-end loop, main program, to demo the effective of interrupt

#---------------------------------------------------------

nop

nop

nop

nop

b Loop 	# Wait for interrupt

end_main:

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~ ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

IntSR:

# Detect which row was triggered

li $t4, 0x1  	# start with row 1

li $v0, 4

la $a0, Message

syscall

detect_row:

sb $t4, 0($t1)  	# assign row to scan

lb $a0, 0($t2)  	# read scan code of key button

bnez $a0, print_key  # if key pressed, print it

sll $t4, $t4, 1  # shift left to check next row

ble $t4, 0x8, detect_row  # loop until all rows checked

print_key:

# Print the key code

li $v0, 34  # print integer (hexa)

syscall

li $v0, 4

la $a0, NewLine

syscall

next_pc:

# Evaluate the return address of main routine

mfc0 $at, $14  # $at <= Coproc0.$14 = Coproc0.epc

addi $at, $at, 4  # $at = $at + 4 (next instruction)

mtc0 $at, $14  # Coproc0.$14 = Coproc0.epc <= $at

return:

eret  # Return from exception

# Assignment 3

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.data

Message: .asciiz "Key scan code "

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# MAIN Procedure

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.text

main:

#---------------------------------------------------------

# Enable interrupts you expect

#---------------------------------------------------------

# Enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

xor $s0, $s0, $s0 	# count = $s0 = 0

Loop:

li $t3, 0x80 # bit 7 = 1 to enable

sb $t3, 0($t1)

#---------------------------------------------------------

# Loop an print sequence numbers

#---------------------------------------------------------

addi $s0, $s0, 1 	# count = count + 1

prn_seq:

addi $v0, $zero,1

add $a0, $s0, $zero 	# print auto sequence number

syscall

prn_eol:

addi $v0, $zero,11

li $a0,'\n' 		# print endofline

syscall

sleep:

addi $v0, $zero,32

li $a0,300 		# sleep 300 ms

syscall

nop 			# WARNING: nop is mandatory here.

b Loop 			# Loop

end_main:

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

#-------------------------------------------------------

# SAVE the current REG FILE to stack

#-------------------------------------------------------

IntSR:

addi $sp, $sp,4 # Save $ra because we may change it later

sw $ra,0($sp)

addi $sp, $sp,4 # Save $ra because we may change it later

sw $at,0($sp)

addi $sp, $sp,4 # Save $ra because we may change it later

sw $v0,0($sp)

addi $sp, $sp,4 # Save $a0, because we may change it later

sw $a0,0($sp)

addi $sp, $sp,4 # Save $t1, because we may change it later

sw $t1,0($sp)

addi $sp, $sp,4 # Save $t3, because we may change it later

sw $t3,0($sp)

#--------------------------------------------------------

# Processing

#--------------------------------------------------------

prn_msg:

addi $v0, $zero, 4

la $a0, Message

syscall

get_cod:

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

li $t3, 0x1 	# check row 4 and re-enable bit 7

polling:

sb $t3, 0($t1) 	# must reassign expected row

lb $a0, 0($t2)  			# read scan code of key button

beqz $a0, next_row  		# if no key pressed, check next row

prn_cod:

li $v0,34

syscall

li $v0,11

li $a0,'\n' 	# print endofline

syscall

next_row:

sll $t3, $t3, 1  			# shift left to check next row

ble $t3, 0x8, polling  		# loop until all rows checked

j next_pc

#--------------------------------------------------------

# Evaluate the return address of main routine

# epc <= epc + 4

#--------------------------------------------------------

next_pc:

mfc0 $at, $14 # $at <= Coproc0.$14 = Coproc0.epc

addi $at, $at, 4 # $at = $at + 4 (next instruction)

mtc0 $at, $14 # Coproc0.$14 = Coproc0.epc <= $at

#--------------------------------------------------------

# RESTORE the REG FILE from STACK

#--------------------------------------------------------

restore:

lw $t3, 0($sp) 	# Restore the registers from stack

addi $sp, $sp, -4

lw $t1, 0($sp) 	# Restore the registers from stack

addi $sp, $sp, -4

lw $a0, 0($sp) 	# Restore the registers from stack

addi $sp, $sp, -4

lw $v0, 0($sp) 	# Restore the registers from stack

addi $sp, $sp, -4

lw $ra, 0($sp) 	# Restore the registers from stack

addi $sp, $sp, -4

return:

eret 		# Return from exception